IONIAN UNIVERSITY – DEPT. OF AUDIO & VISUAL ARTS

LABORATORY OF PHOTOGRAPHY

PROFESSIONAL PHOTOGRAPHY II

INDICATIVE INCREASE OF EXPOSURE (B&W FILM) FOR SHUTTER SPEEDS EQUAL TO OR LONGER THAN 1 SECOND (RECIPROCITY FAILURE COMPENSATION)

(from Kodak Master Darkroom Data Guide for B/W)

Shutter speed

according to light

meter (in seconds)		1	4	12	24	48	100

Multiply shutter

speed value by		1,5x	2x	3x	4x	6x	8x

OR

increase

aperture by		0,5 	1	1,5	2	2,5	3

f-stop	f-stop	f-stops f-stops f-stops f-stops

Corrections for reciprocity failure compensation are different for each film, the above suggestions being only indicative.